BSL – 1 Laboratory Safety

Standard Operating

# Procedure (SOP)

This SOP Applies to EVERYONE Working in

the Laboratory

Date:

Principal Investigator (PRINT NAME):

Department:

Lab Room(s):

Phone:

1. 	Access to the laboratory is limited to staff, or other persons with permission of the

Principal Investigator.

2. 	Persons with cuts or scraps on their hands should wear gloves.   Contaminated gloves must be changed IMMEDIATELY. Under NO CIRCUMSTANCES should gloves be reused.

3.	Laboratory workers must wash hands after handling infectious materials, after removing

gloves, and before leaving the laboratory.

4. 	Laboratory workers must not eat, smoke, handle contact lenses, or apply cosmetics in the laboratory.

5. 	Food for human consumption must be stored outside the laboratory work area.  Food that requires refrigeration must only be stored in a properly labeled refrigerator.

6. 	Only mechanical pipetting devices will be used in the laboratory.

7. 	All sharps must be placed in approved sharps containers. Sharps containers must be autoclaved and discarded when ¾ full.

8.	Forceps, a brush and pan or other mechanical mean should be used to pick up broken glass.  Do not pick up broken glass by hand.  Broken glass must be placed in a sturdy cardboard glass box or sharps container.

9. 	All non-liquid cultures, stocks, and other biologically contaminated waste

must be placed in biomedical waste containers for sterilization and disposal.

10. 	All infectious liquids must be autoclaved or disinfected prior to being poured down a drain.

11. 	All procedures must be performed to minimize creation of splashes or aerosols.

12. 	All work surfaces must be decontaminated at the completion of work, at the end of the

day, or after any spill or splash of viable material.

13. 	Persons who are at increased risk of infection, or for whom infection may have serious

consequences, must not be allowed to enter laboratory when work with infectious agents

is in progress without permission of the Principal Investigator.

14. 	All persons working in laboratory must be advised of potential hazards.

15. 	All support personnel must be trained or made aware of hazards before working in the

laboratory.

16. 	Spills and accidents that result in overt exposures to infectious materials must be

immediately reported to the Principal Investigator and appropriate medical evaluation

must be provided.

17. 	Laboratory equipment, surfaces and other contaminated materials must be

decontaminated with an effective disinfectant on a routine basis, after work with

infectious materials is finished, and especially after overt spills, splashes, or other

contamination.

18. 	Contaminated equipment must be decontaminated before the equipment is repaired in the

laboratory or sent out of the laboratory for repair, maintenance, or other purposes.

19. 	Centrifuges used for high concentrations or large volumes of infectious agents must have

sealed rotor heads or centrifuge safety cups. Safety cups must be opened only in a

Biosafety cabinet.

20. 	Face protection (goggles, mask, face shield, or other splatter guard) must be used for all

procedures when such procedures could produce splashes or sprays of infectious or other

hazardous materials.

21.	Laboratory doors must be locked when the laboratory is not occupied.

22. 	Spaces between benches, cabinets, and equipment must be kept accessible for cleaning.

23. 	Eyewash stations must be flushed at least monthly and kept clear of obstructions.

24. 	All personnel must know the location of the nearest safety shower and must ensure the

area under the shower is kept free of obstructions.

25.  	Emergency contact numbers must be displayed prominently in the laboratory.

26.  	Computers and communication devices (cell phones) must not be used while the user’s hands are gloved.

Print form and continue to next process >>>